Title : Prepare setup for Mininet network emulation environment with the help of Virtualbox and Mininet. Demonstrate the basic commands in Mininet and emulate different custom network topology (Simple, Linear, and Tree).View flow tables.

Name : - Mehatab Mahibub Sanadi

Roll No. : - CO4050

Class : - BE Computer

Subject : - Software Defined Networks

*********************************************************************

Step 1: Install Required Tools (Ubuntu)

sudo apt update

sudo apt install mininet openvswitch-switch git python3 -y

Step 2: Install POX Controller

cd ~

git clone https://github.com/noxrepo/pox.git

cd pox

Step 3: Start POX Controller

sudo python3 pox.py forwarding.l2_pairs openflow.discovery info.packet_dump samples.pretty_log log.level --DEBUG

What this does:

Acts as Layer-2 learning switch

Logs packets and flow installation

Step 4: Create Custom Topology (Remote Controller)

Structure:

3 switches → s1, s2, s3

4 hosts → h1, h2, h3, h4

Controller connected to all switches

Run Mininet with Remote Controller

Open new terminal:

sudo mn --topo tree,depth=2,fanout=2 --controller=remote,ip=127.0.0.1,port=6633

This matches your topology:

s3 → core switch

s1, s2 → edge switches

Each switch → 2 hosts

Step 5: Verify Topology

Inside Mininet:

nodes

net

Step 6: Test Controller

Pingall

What happens:

- First packets go to controller

- Controller installs flows

- Then communication becomes fast

Step 7: Observe POX Logs

Look at POX terminal:

You will see:

- Packet-in messages

- Flow installation logs

- MAC learning

This confirms controller is working

Step 8: View Flow Tables

Inside Mininet CLI:

dpctl dump-flows

Shows:

- Flow entries added by controller

- MAC-based forwarding rules

Step 9: Clear Flow Tables

dpctl del-flows

Step 10: Check ARP Tables

h1 arp

Step 11: Exit & Clean

exit

sudo mn -c

Conclusion:We learn how to install controller and run custom topology using remote controller like POX and floodlight controller. Recognize inserted flows by controller